Patuxent River Navy Flying Club

Instrument Exam

Name:     ___________________________	Date:  _______________________

Instructor:  __________________________Grade: ______________________

- To act as pilot-in-command of PRNFC aircraft under instrument flight rules, an instrument check flight must be flown at least every ______months.

- FAA requires that, to be current to act as PIC under IFR, you must fly at least________ instrument approaches,  _______________procedures, and ____________________and __________________courses through the use of navigation systems within the preceding ________calendar months to meet the requirements of  FAR 61.57.

- Within what period must the following equipment checks have been performed to permit operations under IFR:

transponder & static system __________months (FAR 91.413 and 91.411)

VOR receiver		      __________days (FAR 91.171)

- When checking one VOR receiver against another, the maximum permissible variation           between the two indicated bearings is  +/-______degrees. (FAR 91.171)

- What record shall be made in the aircraft temporary log by the pilot conducting the 30-day VOR receiver accuracy check?  (FAR 91.171)

a.	Date, frequency and bearing reading of the VOR or VOT, tach reading, and signature

b.	Date, place, bearing error and signature

c.	Date, frequency of VOR or VOT, number of flight hours since last check, and signature

d.	Date, place, satisfactory or unsatisfactory, and signature

- What weather conditions must be forecast for at least ______hour(s) before and ______hour(s) after the estimated time of arrival at the intended airport of landing, to eliminate the need for filing an alternate airport?   (FAR 91.169)

____________ft ceiling		____________mile(s) visibility

- An airport without an authorized instrument approach procedure may be included on an IFR flight plan as an alternate, if the current weather forecast indicates that a ceiling and visibility at the ETA will (FAR 91.169):

- be at least 600 feet and 2 miles

- be at least 800 feet and 2 miles

- be at least 1,000 feet and 1 miles

- allow for a descent from MEA, approach, and a landing under basic VFR

- How should you check the altimeter prior to an IFR flight? (“Instr. Flying Hdbk”, Mechanical Altimeter Errors, page 5-4)

- Set the altimeter to 29.92.  With current temperature and the altimeter indication, determine the true altitude to compare with field elevation.

- Set the altimeter to the current temperature.  With current temperature and the altimeter indication determine the calibrated altitude to compare with the filed elevation.

- Set the altimeter first with 29.92 and then the current altimeter setting.  The change in altitude should correspond to the change in setting.

- Set the altimeter to the current altimeter setting.  The indication should be within 75 ft. of the actual elevation for acceptable accuracy.

- According to FAR 91.167, an IFR flight requires that enough fuel be carried to fly to the first   airport of   intended landing, then from that airport to the alternate airport, and then for ________minutes at _________________speed.

- If an alternate is required, at least the following minimums must be forecast per FAR 91.169:

- precision approach:    __________ft ceiling,   ______statute mile(s) visibility

- non-precision approach:   __________ft ceiling,  ______statute mile(s) visibility

- When a pilot elects to proceed to the selected alternate airport, which minimums apply for landing at the alternate?

- IFR alternate minimums.

- 600-1 if the alternate airport has an ILS approach.

- Ceiling 200 ft. above the published minimum; visibility 2 miles.

- The landing minimums for the approach to be used.

- What minimum ceiling and visibility must exist to conduct standard IFR departures from

NHK? (FAR 91.175)

- zero/zero since we operate under FAR 91 as a private flight

- 500 and ½

- 400 and 1

- No lower than authorized landing minimums for which the aircraft is operationally capable of flying.

- When departing from any airport under IFR, to assure obstacle clearance pilots should follow the published d________________     p_________________  (DP) to transition to the enroute structure safely.   (AIM 5-2-8)

- Reports must be made to ATC when: (AIM 5-3-3)

a. vacating an altitude

b. reaching an altitude

c. both

- To comply with ATC instructions for altitude changes of more than 1,000 ft, what rate of climb/descent should be used?  (AIM 4-4-10)

- As rapidly as possible to 500 ft above/below assigned altitude, then at 500 ft/min

- 1,000 ft/min until reaching assigned altitude

- 1,000 ft/min during climb and 500 ft/min during descent

d.	Aircraft optimum rate to 1,000 ft above/below assigned altitude, and then at a rate between 500 ft/min and 1500 ft/min until reaching assigned altitude

- Should you lose two-way radio communication in IMC conditions, what transponder code should you squawk? (AIM 6-4-2)

______________    code

- If you lose two-way radio communication in IMC conditions, which altitude would you fly?  (FAR 91.185)

Circle correct option:     Highest     or    Lowest

of the following altitudes for the segment being flown:

Altitude last assigned

Minimum altitude for IFR operations

Altitude ATC has advised may be expected in a further clearance

- When in a holding pattern, to compensate for wind effect, the drift correction on the outbound leg should be ___________ times the drift correction on the inbound leg.  (AIM 5-3-7)

- When being radar vectored  to an instrument final approach course, if a procedure turn is depicted on the approach chart, it must be performed (AIM 5-4-9):

True/False (circle correct answer)

- When may you obtain a contact approach?  (AIM 5-4-25)

- ATC may assign a contact approach if VFR conditions exist or you report the runway in sight and are clear of clouds.

- ATC may assign a contact approach if you are below the clouds and the visibility is at least 1 mile.

- ATC will only assign a contact approach if you have the runway in sight and VFR conditions exist.

- ATC will assign a contact approach only upon request if the reported ground visibility is at least 1 mile.

- What is the minimum equipment required to execute the NHK VOR/DME RWY 24 approach? (See attached figure).

  - One VOR receiver

  - One VOR receiver and  DME

  - One VOR receiver and Patuxent Radar

  - Two VOR receivers

- Refer to the NHK VOR/DME or TACAN RWY 24 approach chart to answer the following statements.

  - BYNAM, HELEM , and SPRIG are ____________________approach fixes.

  - You should not execute the 11 mile DME arc lower than _______________  ft.  msl.

  - The final approach fix is ___________________ and is _______ DME from PXT.

  - The MAP is   _____________________at  ________ DME from PXT.

- To execute the NHK RNAV (GPS) RWY 14 to the lowest published minimums (400-1) you must have the following. (See attached figure).

  - VOR RNAV receiver

  - GPS WAAS receiver

  - GPS and RNAV receiver

  - PXT Radar assistance and a GPS WAAS receiver

- You should expect to receive glide slope information for the NHK RNAV (GPS) RWY 14 approach if you have an approved GPS WAAS receiver on aboard.

  - True

  - False

- The glide slope for the NHK RNAV (GPS) RWY 14  is a _____ degree glide slope, and the threshold crossing height is ________ ft.